Fundamental of Computer Networks Assignment-1

- List and explain the different types of Ethernet networks.

-10 Mbps :

Original standard using Cat3+ UTP copper or coax;

100m max on twisted-pair.

Suited for basic legacy networks.

​-Fast Ethernet :

100 Mbps over Cat5 UTP (TX, 100m) or multimode fiber (FX, 2km).

Common in older small offices.

-Gigabit Ethernet :

1 Gbps using Cat5e+ UTP (100m) or fiber.

Standard for homes/offices.

-10 Gigabit Ethernet :

10 Gbps on Cat6a UTP (100m) or fiber (SR: 400m; LR: 10km).

For data centers and enterprise backbones.

​

-Multi-Gigabit Ethernet :

2.5/5 Gbps over existing Cat5e/Cat6 (100m).

Bridges Fast/Gigabit for upgrades without recabling.

​

-Higher-Speed Ethernet:

BASE-T (copper) or fiber variants like 100GBASE-SR4 (100m multimode) or 400GBASE-DR4 (500m single mode). Used in cloud/data centers.

-Single-Pair Ethernet :

10/100 Mbps over single twisted-pair; up to 15-40m.

Ideal for automotive/IoT.

- Describe the different types of Ethernet cables based on the cable type and diameter used.

Ethernet Cable Categories Explained Simply

Ethernet cables are grouped into "categories" (Cat1, Cat2, etc.) based on how well they handle data speed and interference. The main differences come from their bandwidth (signal cycles per second in MHz), maximum data rate (bits per second), and shielding to block noise.

Early Categories (Voice/Legacy Use):

-   Cat1  :

Basic unshielded twisted pair (UTP) for phone lines only no data networking.

Two wires twisted together for analog voice.

-   Cat2  :

UTP for early data like 4 Mbps IBM Token Ring networks in the 1980s still voice-capable.

-   Cat3  :

First for Ethernet (10 Mbps over 100m).

four UTP pairs, common in 1990s buildings but too slow now.

-   Cat4  :

UTP upgrade to 16 Mbps for Token Ring.

found in old setups where rewiring costs too much.

Modern Categories (Data Networking):

-   Cat5  :

UTP with 100 MHz bandwidth for 100 Mbps Fast Ethernet up to 100m;

handles data, video, phones.

-   Cat5e   :

Improved twists reduce crosstalk for 1 Gbps Gigabit Ethernet;

standard for new basic installs.

-   Cat6  :

250 MHz with optional shielding and a plastic spline separator;

1 Gbps at 100m or 10 Gbps up to 55m.

-   Cat6a  :

500 MHz fully shielded;

reliable 10 Gbps over 100m by cutting "alien" crosstalk between pairs.

-   Cat7  :

600 MHz with braid-and-foil shielding (S/FTP);

10 Gbps at 100m, uses proprietary connectors; good for noisy areas.

-   Cat7a  :

Boosted to 1 GHz for 40 Gbps short distances (50m);

rare due to non-standard status.

-   Cat8  :

Top-tier 2 GHz (2000 MHz) with heavy full shielding;

25-40 Gbps up to 30m for data centers/switches;

rigid and thick.

3) Differentiate between UTP and STP cables.

Unshielded Twisted Pair (UTP): UTP cables lack additional shielding, making them lightweight, flexible, and cost-effective. They are commonly used in home and office networks where interference is minimal. UTP cables are easy to install and terminate, making them ideal for small-scale setups.

Key features of UTP

-UTP cables are cheaper than STP cables.

-Their flexibility simplifies routing and termination.

-UTP cables are more susceptible to EMI and crosstalk, limiting their use in high-interference environments.

Suitable for short distances (up to 100 meters) with speeds ranging from 10 Mbps to 1 Gbps, depending on the category.

Shielded Twisted Pair (STP)

STP cables include an additional shielding layer, such as metal foil or braided copper, to protect against EMI and crosstalk. This makes them suitable for industrial environments or areas with high interference.

Key features of STP:

- The shielding reduces signal degradation and ensures data integrity.

- STP supports longer distances (up to 1000 meters) and higher speeds, making it ideal for high-bandwidth applications.

- STP cables are bulkier, require grounding, and are more challenging to terminate.

-They are more expensive due to the added shielding and installation complexity.

UTP: Best for home and office networks, patch cables, and short LAN runs where interference is minimal.

STP: Ideal for industrial settings, data centers, and high-speed networks where EMI is a concern, such as near motors or high-voltage equipment.

4. Explain network topologies and provide one example for each?

Ans.

- Bus Topology: is a type of network where all computers and devices are connected to one main cable called the backbone.

- Data can travel in both directions (bi-directional).

- It is a multi-point connection because many devices share the same cable.

- It is not very reliable because if the main cable fails, the whole network stops working.

In Bus-topology Ethernet, only the CSMA/CD method is used to control how devices send data.

ex:- Ethernet LAN where coaxial cable is used to connect devices.

- Ring Topology: Ring topology is a network where all computers are connected in a circle (like a ring). Each computer is connected to two other computers. Data travels in one direction around the circle.

ex:- IBM token ring, ring based fiber network.

- Mesh Topology: Mesh topology is a network where each device is connected to many or all other devices directly. So, there are multiple paths to send data. This means if one connection fails, data can take another path.

ex:- Army communication system use mesh system.

- Star Topology: is a network where all devices are connected to one central device (like a hub or switch). All communication goes through the central device.

ex:- School computer lab all computer connected to one central switch.

- Hybrid Topology: Hybrid topology is a network that combines two or more different types of topologies (like star, ring, bus, mesh) in one system. It is used in large networks to get the advantages of different topologies together.

ex:- Internet Infrastructure.

5) Types of Mesh Topology?

Mesh Topology is a network configuration where every device is interconnected with every other device, providing multiple route for data to travel. The nodes are connected to each other completely via a dedicated link during which information travels from nodes to nodes. If a mesh network has N nodes, then there are N(N-1)/2 links. Each computer not only sends its signals but also transfer data from other computers. The connections within the mesh are often wired or wireless. In this article, we will discuss the Mesh Topology in detail.

Full Mesh Topology

All the nodes within the network are connected with every other If there are n number of nodes during a network, each node will have an n-1 number of connections. A full mesh provides an excellent deal of redundancy, but because it is expensive to implement, it's usually reserved for network backbones. Total number of links required for the mesh topology is [n(n-1)]/2.

Partial Mesh Topology

The partial mesh is more practical as compared to the full mesh. In a partially connected mesh, all the nodes aren't necessary to be connected with one another during a network. Peripheral networks are connected using partial mesh topology.

Applications of Mesh Topology:

Wireless Networks: Many homes and small office spaces uses Mesh Wi-fi System for better internet coverage and connectivity.

Industrial and Manufacturing Networks: Many industries use of mesh topology to control machinery, ensuring better productivity.

Smart Homes: Some advanced homes also use mesh topology for connecting home appliances like smart lights, security systems, etc.

Military Communication: Mesh Topology are used in Military Purposes as if one connection fails, then there is always a chance of another connection.

6) Types of cables?

Ethernet cables used in LANs

Coaxial Cables

Twisted Pair Cables

Fiber optic Cables

Coaxial Cables: A coaxial cable is used to carry high-frequency electrical signals with low losses. It has a copper conductor in the middle that is surrounded by a dielectric insulator generally made of PVC or Teflon.

Types:

Baseband: Which is used for digital transmission. It is mostly used for LAN's. Baseband transmits a single signal at a time with very high speed.

Broadband: This uses analog transmission on standard cable television cabling it transmits several simultaneous signal using different frequencies.

Twisted Pair cable : A twisted pair is a copper wire cable in which two insulated copper wires are twisted around each other to reduce interference or crosstalk. It uses 10BASE-T, 100BASE-T.

Types:

Shielded Twisted Pair (STP) Cable: In STP the wires are covered by a copper braid covering or a foil shield, this foil shield adds a layer that protects it against interference leaking into and out of the cable. Hence, they are used for longer distances and higher transmission rates.

Unshielded Twisted Pair (UTP) Cable: Unshielded twisted pair cable is one of the most commonly used cables in computer networks at present time. UTP consists of two insulated copper wires twisted around one another, the twisting of wires helps in controlling interference.

Fiber Optic Cable: Fiber optic cables use optical fibers which are made of glass cores surrounded by several layers of covering material generally made of PVC or Teflon. It transmits data in the form of light signals due to which there are no interference issues in fiber optics. It uses 10BaseF, 100BaseFX.

Single mode optical fiber: Designed to carry only a single mode of light - transverse mode. Used for long-distance transmission of signals. Single-Mode cables support the transmission of single light mode with Brighter & Higher-power sources with Less attenuation.

Single-mode fiber can carry signals at a much greater speed. Because of the smaller core size, termination is more difficult.

Applications: Telecom, CATV networks.

Multi-Mode Optical Fiber: Designed to carry multiple modes of light at once. Used for comparatively shorter distance transmission of signals. Multi-Mode cables support the transmission of multiple light modes with-Less brightness, High attenuation.

There is much more degradation of signals. Because of the larger core size, termination is easier.

Applications: Security systems, LANs.